Stitchi 1.0 — Developer Guide

Architecture, setup, configuration, and MATLAB/UDP integration

# 1. Requirements and environment

The supplied project was developed/tested in the following environment. These are the reported installed versions, not a guarantee that every version is required or that the same combination will work unchanged on every machine.

| Component | Reported version / setting |
| --- | --- |
| Operating environment | Windows PowerShell; virtual environment shown as `(venv)` |
| Python | 3.11.4 |
| MediaPipe | 0.10.9 |
| OpenCV | opencv-python 4.13.0.92; opencv-contrib-python 4.13.0.92 |
| NumPy | 2.4.2 |
| Intel RealSense Python API | pyrealsense2 2.55.1.6486 |
| Other notable packages | protobuf 3.20.3; absl-py 2.4.0; attrs 25.4.0; Pillow 12.1.1; requests 2.34.2; jsonschema 4.26.0 |
| MATLAB | Must support `udpport`, `jsonencode`, `jsondecode`, and the string APIs used by the scripts. The exact MATLAB release was not provided. |
| Hardware | A camera compatible with the chosen input mode; for depth-based operation, an Intel RealSense camera and compatible RealSense SDK/driver. A network connection between the video-system PC and audio-system PC is required for remote UDP communication. |

You can just run `python -m pip install -r requirements.txt` to install all packages in python or its virtual environment. The `requirements.txt` contains all library names and versions.

All of the other packages listed by pip are indirect dependencies that should automatically installed alongside the main libraries and are required for their functionality. The full pip list supplied with the project also included: asttokens 3.0.1, backcall 0.2.0, beautifulsoup4 4.15.0, bleach 6.4.0, certifi 2026.6.17, cffi 2.0.0, charset-normalizer 3.4.7, colorama 0.4.6, contourpy 1.3.3, cycler 0.12.1, decorator 5.3.1, defusedxml 0.7.1, docopt 0.6.2, executing 2.2.1, fastjsonschema 2.21.2, flatbuffers 25.12.19, fonttools 4.61.1, idna 3.18, IPython 8.12.3, jedi 0.20.0, Jinja2 3.1.6, jsonschema-specifications 2025.9.1, kiwisolver 1.4.9, MarkupSafe 3.0.3, Matplotlib 3.10.8, mistune 3.3.2, nbclient 0.11.0, nbconvert 7.17.1, nbformat 5.10.0, packaging 26.0, pandocfilters 1.5.1, parso 0.8.7, pickleshare 0.2.2, pip 26.1.2, pipreqs 0.5.0, platformdirs 4.10.0, prompt-toolkit 3.0.52, pure-eval 0.2.3, Pygments 2.20.0, pycparser 3.0, pyparsing 3.3.2, python-dateutil 2.9.0.post0, pyzmq 27.1.0, referencing 0.37.0, rpds-py 2026.6.3, setuptools 65.5.0, six 1.17.0, SoundDevice 0.5.5, soupsieve 2.8.4, tornado 6.5.7, traitlets 5.15.1, typing_extensions 4.16.0, urllib3 2.7.0, wcwidth 0.8.2, webencodings 0.5.1, yarg 0.1.9.

# 2. Project structure

| File | Responsibility |
| --- | --- |
| main.py | Application orchestration and frame loop; connects all subsystems. |
| settings.py | Central configuration read by the other modules. |
| frame_handler.py | Selects, initializes, reads, displays, and closes camera/video/depth sources. |
| MediaPipe_handler.py | Pose and hand landmark inference, visibility/movement data, hand ROI handling. |
| coordinates_handler.py | Pixel/depth-to-3D conversion and coordinate compensation. |
| angle_handler.py | Arm vector, azimuth/elevation, room ray tracing, angle smoothing and special camera-pointing case. |
| gesture_handler.py | Interprets landmarks as grab/release transitions and control gestures. |
| GUI.py | Instrument tiles and controls; applies selection, grab, movement, release and GUI state. |
| communication.py | UDP/JSON sender, receiver thread, message queue, send-on-change logic. |
| own_functions.py | Shared buffers, movement/aperture helpers, drawing, mouse, file and geometry utilities. |
| analyse.py | Test-data capture, file discovery, status annotation, JSON/list persistence, and offline comparison. |
| matlab-code/send_udp.m | Send-only instrument-list smoke test. |
| matlab-code/receiv_udp.m | Receive-only UDP diagnostic. |
| matlab-code/UDP_send_instruments_then_receiv_orders.m | Send a fixed instrument list, then print incoming messages. |
| matlab-code/UDP_send_instruments_per_comandline_and_reseiv.m | Interactive instrument-add test and receive loop. |
| matlab-code/README.md | Short original German instructions for starting Python and MATLAB communication. |

# 3. Runtime call flow

Running `python main.py` enters the `if __name__ == '__main__'` block and calls `main()` using values from `settings.py`.

`main()` creates `GuiOverlay`, `FrameSource`, two `HandDetector` instances, `PoseDetector`, `RoomAngleDetector`, `GestureDetector`, `SendOnChange`, and `SaveUdpToDict`. The receiver starts in a background thread.

`FrameSource` reads each frame. In RealSense mode it can provide aligned colour and depth frames; in ordinary webcam/video mode only the image is available.

`PoseDetector` estimates body landmarks and exposes shoulder/hand reference points. `HandDetector` estimates hand landmarks. The results are passed to `GestureDetector` and the angle module.

`RoomAngleDetector` estimates pointing azimuth/elevation from hand and shoulder coordinates. With RealSense depth it uses depth/intrinsics; other paths use available MediaPipe world coordinates or fallback approximations.

`GestureDetector` detects hand open/closed states and transitions. A transition from open to closed starts a grab; reopening releases the currently grabbed GUI item. Additional gestures trigger GUI actions.

`GuiOverlay` uses the hand positions and grab/release events to select and move instrument tiles. The selected instrument state is exposed by `get_info()`.

`SendOnChange` sends a JSON/UDP message when the GUI instrument state changes. Separately, `SaveUdpToDict` receives messages such as `add_instruments` from MATLAB and places them in a queue for `main()` to process.

On exit, `main()` closes the frame source and prints/saves available analysis statistics.

# 4. Module interaction summary

The live dependency chain is approximately:

main.py
  ├─ frame_handler.FrameSource  → camera / video / RealSense frames
  ├─ MediaPipe_handler.PoseDetector + HandDetector → landmarks
  │    └─ coordinates_handler → 3D conversion / camera-room compensation
  ├─ gesture_handler.GestureDetector → grab/release and control triggers
  ├─ angle_handler.RoomAngleDetector → azimuth/elevation in room coordinates
  ├─ GUI.GuiOverlay → instrument selection and movement
  └─ communication.SendOnChange / SaveUdpToDict ↔ MATLAB audio-system process

Shared helpers: own_functions.py
Test and analysis support: analyse.py
Shared configuration: settings.py

# 5. Starting the application

Install Python 3.11.4 and the compatible Intel RealSense SDK/driver if using a RealSense camera. Connect the camera before launching the program.

Open PowerShell in the project root (`Stitchi1.0`) and activate the project virtual environment.

Install the project dependencies in that environment. Prefer a pinned requirements file for reproducibility; the archive did not contain one, so use the package versions listed in Section 1 as the recorded environment.

Check `settings.py`: input source, room/camera geometry, network IP addresses, UDP port, and GUI options.

Run `python main.py`. The configured camera/video window should open. Use `ESC` to exit.

The current `video_source = 'depth'` setting selects the RealSense depth camera path. For an ordinary webcam, set it to an integer camera index such as `0`. For a video file, set it to the filename/path. RealSense recordings use `.bag`; the configured `rs_save_type` is `.bag`.

# 6. settings.py — parameter reference

The following table explains the practical effect of each setting present in the supplied file. Values in parentheses are the values in the archive. Changes can interact; tune one group at a time and validate with repeatable videos.

| Setting | Current value | Effect |
| --- | --- | --- |
| name / version | `Gesture control` / `Stitchi 1.0` | Window/application label only. |
| fps | 30 | Processing frame-rate cap and requested camera frame rate. Higher values increase responsiveness only if the camera and computer can sustain them; lower values reduce load but add latency. |
| start_frame / end_frame | 1 / 0 | Video playback start and optional ending frame. `end_frame = 0` disables an explicit end limit. |
| live_stream_resulutuin | (1280, 720) | Requested live image dimensions. Higher resolution can improve landmark detail but increases processing load. Must be supported by the selected camera. |
| window_size | (1920, 1080) | GUI/mouse coordinate reference and display size; changing it can affect mouse mapping and layout. |
| hand_width_factor / hand_opening_factor | 1.2 / 1.3 | Scaling factors used by hand/gesture sizing logic; may change hand-region/aperture interpretation and should be tuned with recorded examples. |
| skip_frames | 100 | Frame skip/analysis-related parameter; verify its call sites before relying on it for live recognition because it is not a general frame-rate setting. |
| icon_folder / screenshots_folder / gui_info_window_image_path | relative paths | Locations of GUI assets and screenshots. Paths are relative to the working directory; missing assets can affect icons/info visuals. |
| video_folder | relative test-video folder | Location searched for offline test videos and RealSense recordings. |
| port | 5005 | UDP destination/listener port used by the Python communication classes. Must match MATLAB target/local ports. |
| IPv4_audiosystem / IPv4_videosystem | 192.168.0.10 / 192.168.0.11 | IP addresses of audio-system PC and video-system PC. Replace with the actual addresses on the current network. |
| bind_ip | 0.0.0.0 | Python UDP receiver binds to all local IPv4 interfaces. Use `127.0.0.1` only for same-machine tests. |
| printout_udp | True | Enables console output for UDP messages when the call requests printing. |
| add_inst_key / start_instruments | `add_instruments` / empty list | JSON key used to add instruments; optional initial list inserted at startup. |
| video_source | `depth` | Selects RealSense live depth camera (`depth`), a standard camera index (`0`, `1`, …), an ordinary video file, or a RealSense `.bag` recording. |
| default_buffer_mode | `median` | Default buffer aggregation mode. Median filtering can reduce outlier jitter; smoothing can add lag. |
| cam_inclination_angle | 0 degrees | Camera inclination compensation used in 3D coordinate conversion. Set to the calibrated physical tilt; an incorrect value biases pointing angles. |
| visibility_threshold | 0.92 | Minimum landmark visibility used by pose/point checks. Higher values reject uncertain points more often; lower values may increase detections but admit noisy landmarks. |
| align_depth | True | Aligns RealSense depth to the colour stream, so landmark pixels can be sampled from corresponding depth coordinates. Usually needed for this workflow. |
| active_hand | `both` | Hand selection mode: `right`, `left`, `moving`, `top`, or `both`. `moving` chooses based on motion; `top` chooses the highest image hand; `both` processes both hands. |
| no_hand_frame_count | 50 frames | Frames without a detected hand before treating it as absent. At 30 FPS, 50 frames are about 1.67 s; increasing it delays loss/reset recognition. |
| moving_min_speed | 150 px/s | Minimum image-space speed used to classify a hand as moving in the moving-hand mode. |
| moving_check_time | 0.3 s | Time interval over which hand motion is assessed. Longer intervals smooth speed estimates but respond more slowly. |
| hand_change_min_speed | 200 px/s | Speed threshold for switching the selected hand in `moving` mode. Higher values reduce accidental switching but may miss slower hand changes. |
| use_roi | True | Restricts hand detection to a region around the expected hand. Can improve speed/detection in the intended region, but may miss a hand outside the ROI. |
| roi_min_size | 50 px | Minimum ROI dimension. Larger minimum regions can help retain enough context but increase image area processed. |
| hand_shoulder_intersect_dist | 0.10 m | Depth-camera threshold for treating hand and shoulder as too close in image/depth geometry, used for the special case where the arm points toward the camera. |
| hand_shoulder_intersect_pixel | 20 px | Image-space threshold for the same special case when depth-based distance is unavailable. Despite the source comment, the unit is pixels, not metres. |
| hand_center_hands | (5, 17) | MediaPipe Hands landmark indices used to estimate hand centre. Changing indices changes the point used for cursor/pointing calculations. |
| left_hand_center_pose / right_hand_center_pose | (17,19) / (18,20) | MediaPipe Pose landmark pairs used as hand-centre approximations when using pose landmarks. |
| grab_detection_methode | `dynamic` | Selects hand aperture method (`static` or `dynamic`). The chosen method affects how open/closed states are inferred and therefore GUI grab/release reliability. |
| dynamic_grab_distance_difference | 1 | Minimum aperture/distance change for dynamic open/closed detection. Larger values require a stronger change; smaller values are more sensitive and may trigger false changes. A value of 1 means that the opening distance must double or halve to trigger a change. A value of 0.5 means that it must increase by a factor of 1.5 or decrease to 0.75 of its original value. |
| dynamic_grab_time_difference | 1 second | Time interval for dynamic aperture comparison. Longer intervals are less sensitive to fast changes; shorter intervals react faster but may be noisier. |
| hand_status_buffer_size / hand_status_buffer_atleast | 3 / 3 frames | Number of recent hand-state samples and minimum agreeing samples for buffered/static classification. More smoothing reduces flicker but can delay transitions. For static hand detection method. |
| hand_open_thrashold / hand_close_thrashold | 65 / 75 | for the static hand detection method, values ​​between zero and 100 determine the degree of hand opening at which a hand transitions to a state of being "too open" or "too closed." |
| angle_buffer_size | 5 frames | Number of recent angle samples used for smoothing. Larger buffers stabilize pointing but add lag. |
| volume_decimal_place | 2 | Number of decimal places used when quantizing volume values sent or displayed. |
| real_depth_angle_resulutuin | 15 degrees | Quantization step/resolution for room pointing angles when depth is used. Coarser steps reduce jitter but reduce directional precision. |
| ray_tracing_step_size | 0.1 m | Step distance for ray tracing to the room boundary. Smaller steps can improve boundary resolution but require more iterations. |
| room_size | (3.4, 2.7, 3.4) m | Room dimensions in x/y/z used for ray tracing. Must match the coordinate convention and modeled room. |
| room_center | (1.7, 1.5, 1.7) m | Reference centre of the room used for room-relative direction calculations. |
| cam_pos_in_room | (1.7, 1.7, -0.1) m | Camera position in room coordinates. Errors here bias the reconstructed pointing direction. |
| estimated_hip_hight | 0.95 m | Estimated hip height used to offset MediaPipe world landmarks into the room coordinate system. |
| show_gui | True | Enables/disables the instrument GUI overlay and related GUI-state sending. |
| gui_tile_max_size | 100 px | Maximum instrument tile size used by GUI layout. |
| gui_hight | 1/4 of frame height | GUI panel height proportion relative to the video frame. |
| controll_trigger_text_time | 1.0 s | Duration of temporary on-screen feedback after a control gesture/key. |
| instrument_start_volume | 0.5 | Initial volume for new instrument tiles, on a 0–1 scale. |
| no_mouse_control | False | When True, disables mouse selection/grab/release input; gesture control remains available. |
| mirrow_view | True | Mirrors the displayed image/coordinate handling. Must stay consistent with landmark/depth coordinate interpretation. |
| draw_aperture / draw_hand_skeleton / draw_body_skeleton / draw_roi / draw_hand_vlidation_dist | True | Debug overlays for aperture, hand/body landmarks, ROI, and hand-validation distance. Useful for tuning; overlays may add display/rendering overhead. |
| show_depth_frame | False | Controls display of the depth image for debugging. |
| type_instrument | `instrument` | Type field used to identify outgoing instrument-control messages. |
| rs_save_type | `.bag` | File extension used to identify RealSense recordings. |
| display_feedback_mode | two modes: processing+GUI; GUI only | Cycle of visual feedback modes. Add/remove valid mode strings only if the GUI code supports them. |
| red / blue / green / white / black / yellow | BGR colour tuples | Colours for overlays and GUI feedback; OpenCV uses BGR ordering. |

# 7. Adjust hand grabbing detection

Use ‘grab_detection_methode’ to set the static or dynamic hand-detection method. If you wish to modify the dynamic hand-detection method, focus on parameters ‘dynamic_grab_distance_difference’ and ‘dynamic_grab_time_difference’.

If you wish to modify the static hand-detection method, focus on parameters ‘hand_status_buffer_size’ , ‘hand_status_buffer_atleast’, ‘hand_open_thrashold’ and ‘hand_close_thrashold’.

These are described in greater detail in section 6.

# 8. MATLAB audio-system communication (UDP/JSON)

The video system performs gesture recognition and GUI interaction; the MATLAB/audio-system process receives instrument state and can send instrument names back to populate the GUI. UDP is connectionless: the sender does not establish a session or receive a delivery guarantee. JSON is the payload format.

## 8.1 Addressing and ports

In `settings.py`, `IPv4_audiosystem = 192.168.0.10` is the Python sender's target, `IPv4_videosystem = 192.168.0.11` is the video PC address used by MATLAB scripts, and `port = 5005` is the shared destination/listener port.

The Python receive thread binds to `bind_ip:port`; with `bind_ip = 0.0.0.0`, it listens on all local IPv4 interfaces. MATLAB scripts bind their local `udpport` to port 5005 and send to `192.168.0.11:5005`.

Using port 5005 on both computers is valid: the local IP addresses differ. On one computer, two independent sockets cannot normally bind exclusively to the same local IP/port. If both apps run on one PC, configure a single listener or use different local ports.

Ensure both PCs are on a reachable network, use the correct IPv4 addresses, and allow inbound UDP port 5005 in the Windows firewall. A successful `write()` call does not prove that the other application received the packet.

## 8.2 Message flow

Start `python main.py` on the video-system PC. It creates `SaveUdpToDict`, which starts a background receive loop, and `SendOnChange`, which sends changed GUI instrument state to the audio-system IP.

Run a MATLAB sender/receiver script on the audio-system PC. It sends JSON such as `{"add_instruments":["piano","violin"]}` to the video PC.

The Python receive loop decodes the JSON and queues it. `main.py` reads the queued `add_instruments` field and calls `gui.add_instrument(...)` for each name.

When an instrument is selected and moved/released, the GUI exposes its current state. `SendOnChange` serializes and sends the changed state as JSON to the audio-system PC.

MATLAB reads incoming datagrams, converts UTF-8 bytes to text, applies `jsondecode`, and prints the decoded structure. Replace the print statement with the audio engine's parameter-update function when integrating the real audio system.

## 8.3 Recommended communication test

Check and record the actual IPv4 addresses on both PCs. Update `IPv4_audiosystem`, `IPv4_videosystem`, `targetIp`, `targetPort`, and `localPort` as needed.

Temporarily allow UDP port 5005 through the firewall on both systems.

Start Python `main.py` first. Confirm that the video window opens and no port-binding exception appears.

Run `UDP_send_instruments_then_receiv_orders.m` in MATLAB. The instrument names should appear in the GUI; MATLAB should print any UDP messages sent back by Python when instrument state changes.

Select an instrument using the hand gesture, move the pointing direction, and release it. Confirm that the received JSON fields and angle values change as expected.

If testing on a single PC, use `127.0.0.1` (special IPv4 loopback address) and ensure only one application owns each local listener port. For separate PCs, use the actual LAN IPs.

If nothing arrives, verify IP/port, firewall, that the Python receiver thread is running, and that no other process already occupies local UDP port 5005. On Windows, `netstat -ano | findstr :5005` helps identify the PID holding the port.

## 8.4 Which MATLAB script should be used?

| Script | Purpose |
| --- | --- |
| send_udp.m | One-shot send-only smoke test; it does not bind local port 5005 or receive replies. |
| receiv_udp.m | Receive-only diagnostic on local port 5005. |
| UDP_send_instruments_then_receiv_orders.m | Recommended basic round-trip test: sends a fixed instrument list and continuously receives responses. |
| UDP_send_instruments_per_comandline_and_reseiv.m | Interactive test: adds one instrument name per prompt and receives responses. Since input() blocks, received messages are only processed after the user returns to the loop. |

# 9. Adding instruments and audio-system connection

The available instrument tiles can be provided by the MATLAB/audio-system application over UDP. When the communication is configured, the audio system sends a JSON message using the key `add_instruments`, for example an array containing instrument names. For example: ["trumpet", "trumpet2", "piano", "violin","drum","r2d2"]. Each name can appear only once. The names are then added to the GUI. If no instruments appear, check network addresses, port 5005, firewall rules, and the MATLAB sender must be configured first.

Instruments can also be pre-configured by the state in the settings.py file under `start_instruments`.

The 'icons' folder contains the image files for all GUI tiles. When a new instrument is added, the system automatically checks whether a suitable image for that instrument exists in this folder. It selects the first image that includes the full instrument name in its title. You can add images there. If you do so, make sure they are square and measure approximately 100x100 to 300x300 pixels.

# 10. Adjrust GUI menu tiles and information

The image ‘info_window.png’ contains the information displayed in the info window, which is opened either by the info gesture, info button or by pressing the I key. The icons for the instrument menu and the info button are located in the icons folder. If you want to adjust any of them, in particular the info window, it is recommended to use the raw files in the ‘appendix’ folder to create new PNG files. I used the free image editing software ‘panit.net’.

The volume bar is generated dynamically from text within the ‘VolumeBar’ class in the ‘GUI.py’ file.

# 11. Operational caveats

The supplied Python source starts a receive thread and calls a receive function that binds to port 5005. Do not start multiple application instances or multiple receive loops on the same local port.

UDP can drop or reorder packets. If the audio system needs guaranteed delivery or state recovery, add sequence numbers/acknowledgements or periodic full-state updates.

The current MATLAB scripts are diagnostics: they print JSON but do not implement actual audio synthesis/control.